Assignment

Background

The floppy disk contains a demonstration version of the Lanwatch (R) network

analyzer program. This version of the program was obtained from FTP Software,

which has since transferred rights for the commercial version to Precision

Guesswork.

Your floppy disk contains the following files:

[.]             [..]            1.bat           2.bat           3.bat

4.bat           5.bat           6.bat           7.bat           8.bat

9.bat           a.bat           assign.doc      assign.txt      DEMO-LW.EXE

DEMO.TXT        ETHHOST.TXT     ftp.dmp         ftp.lwd         HOSTS.TXT

http.dmp        http.lwd        lanwatch.doc    lanwatch.txt    menu.bat

shirley.dmp     shirley.lwd     singapore.txt   smtp.dmp        smtp.lwd

telnet.dmp      telnet.lwd      unix.doc        unix.txt        windows.doc

windows.txt

This version of the Lanwatch program (DEMO-LW.EXE) will not capture packets

from a network. However, it will display files such as shirley.lwd that

have been captured with the full version of Lanwatch.

In addition to DEMO-LW.EXE, the floppy contains a variety of dump (.lwd)

files.  To examine the dump files, get to a Microsoft "Command" or "DOS"

prompt and enter "MENU" to run MENU.BAT. This command file describes

other command files (1.bat to 9.bat and a.bat to b.bat) that you can run

by entering the appropriate number or letter.

As the menu produced by MENU.BAT indicates,

Enter 1 for lanwatch.txt - to display a copy of the lecture notes

Enter 2 for shirley.lwd  - the dump described in class (shirley to orst)

Enter 3 for ftp.lwd      - FTP from 192.168.1.101 to 155.247.166.104

Enter 4 for http.lwd     - HTTP from 192.168.1.101 to 155.247.166.104

Enter 5 for telnet.lwd   - Telnet from 192.168.1.101 to 155.247.166.104

Enter 6 for smtp.lwd     - SMTP from 192.168.1.101 to 204.127.198.27

Enter 7 for singapore.txt- traceroute to www.nus.edu.sg

Enter 8 for unix.txt     - summary of unix networking commands

Enter 9 for windows.txt  - summary of windows networking commands

Enter a for assign.txt   - YOUR HOMEWORK ASSIGNMENT

Since DEMO-LW.EXE cannot capture packets, network administrators are not

likely to object to its use. Ethereal (http://www.ethereal.com) is a

superior packet analysis program that will capture and display packets.

The .dmp files on the floppy disk are copies of the .lwd files in Ethereal

format (also called TCPdump format). Since Ethereal can capture packets from

a network, it should not be installed on a company machine without talking

to the network and system managers. If you want to install Ethereal on a

home PC, a binary version is available. If you actually want to capture

packets sent and received by your home PC, you will need to install the

packet capture software called WinPcap (http://winpcap.polito.it/) in

addition to Ethereal.

Assignment

1. From a command prompt, run 3.bat from the floppy disk and answer the

following questions about the ftp transfer. (When you start DEMO-LW.EXE,

it will only display a summary line for each packet. Press the "Insert"

key to see detailed information about the packet. If the hex dump of the

packet does not fit on the screen, use the "Tab" key to scroll through

the hex dump.)

a. What is the IP address of the client?                ____.____.____.____

b. What (decimal) ports did the FTP client use for FTP?          ____  ____

c. What is the name of the FTP server?     ________________________________

(Hint - use nslookup)

d. What is the IP address of the server?                ____.____.____.____

e. What (decimal) ports did the FTP server use for FTP?          ____  ____

f. What account name was used to log on?   ________________________________

g. What password was used?                 ________________________________

h. What file was transferred?              ________________________________

2. From a command prompt, run 4.bat from the floppy disk to see the dump

produced when 192.168.1.101 requested a web page from 155.247.166.104

a. What was the URL (beginning with http://) used to request the web page

(Hint - you will need to use the tab key, not nslookup).

______________________________________________________________________

b. What (decimal) port did the HTTP client use for the request?    ______

c. What operating system does the client

claim to be (apparently a Microsoft

internal name for Windows XP)?          ______________________________

e. What version of HTTP is the client using

(this version requires the "Host:" header line)?                ______

e. What operating system is the server

runnning?                               ______________________________

f. What is the name and version of the

server software (a big help if you

are trying to hack your way in)?        ______________________________

g. What (decimal) port did the HTTP server use?                    ______

3. From a command prompt, run 5.bat from the floppy disk to see the dump

produced when 192.168.1.101 initiated a telnet session to 155.247.166.104

a. What (decimal) port did the telnet client use?                  ______

b. What (decimal) port did the telnet server use?                  ______

c. What login name was used?               ______________________________

d. What password was used?                 ______________________________

e. What operating system is the server

running?                               ______________________________

f. Why does each letter I type appear in two separate packets?

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

4. From a command prompt, run 6.bat from the floppy disk to see the dump

produced when 192.168.1.101 initiated an smtp session to 204.127.198.27

a. What (decimal) port did the telnet client use for the DNS

query (note the UDP, not TCP, is being used)?                   ______

b. What (decimal) port did the DNS server use (it is listed in

/etc/services)?                                                 ______

c. How many "A" records are retuned for smtp.comcast.net?          ______

d. What is the IP address of the first nameserver

for comcast.net (easy)?                            ____.____.____.____

e. What (decimal) port did the SMTP client use?                    ______

f. What (decimal) port did the SMTP server use

(in /etc/protocols of course)?                                  ______

g. Who is the mail from ("MAIL From:")   ________________________________

h. Who is the mail to? ("RCPT To:")      ________________________________

i. What is the magic word?               ________________________________

5. All of the conversations above were initiated by 192.168.1.101. Why can't

you find a DNS entry (like stafford123.comcast.net) for this IP address?

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

6. The following output was produced when I executed the command

"tracert www.nus.edu.sg" (8.bat will produce the same output).

C:\> tracert www.nus.edu.sg

Tracing route to nusinfo.nus.edu.sg [137.132.12.124]

over a maximum of 30 hops:

1   3 ms   1 ms   1 ms  155.247.107.1

2   4 ms   5 ms   4 ms  155.247.4.1

3   5 ms   4 ms   5 ms  conwellbln2.is.temple.edu [155.247.119.226]

4   5 ms   5 ms   5 ms  local.temple.magpi.net [198.32.42.81]

5   5 ms   5 ms   5 ms  phl-02-08.backbone.magpi.net [198.32.42.197]

6   8 ms   7 ms  18 ms  remote1.abilene.magpi.net [198.32.42.210]

7  12 ms  22 ms  11 ms  nycmng-washng.abilene.ucaid.edu [198.32.8.84]

8  32 ms  32 ms  36 ms  chinng-nycmng.abilene.ucaid.edu [198.32.8.82]

9  44 ms  44 ms  36 ms  iplsng-chinng.abilene.ucaid.edu [198.32.8.77]

10  68 ms  71 ms  61 ms  kscyng-iplsng.abilene.ucaid.edu [198.32.8.81]

11  56 ms  55 ms  55 ms  dnvrng-kscyng.abilene.ucaid.edu [198.32.8.13]

12  81 ms  81 ms  96 ms  sttlng-dnvrng.abilene.ucaid.edu [198.32.8.49]

13  81 ms  82 ms  81 ms  pnw-gigapop-b.nus.edu.sg [198.32.171.47]

14  81 ms  81 ms  80 ms  ge3-0-0.seattle-cr1.singaren.net.sg [202.3.135.1]

15 239 ms 239 ms 240 ms  pos1-0.pgp-cr1.singaren.net.sg [202.3.135.6]

16 239 ms 239 ms 240 ms  ge3-9.pgp-dr1.singaren.net.sg [202.3.135.18]

17 239 ms 239 ms 239 ms  nus-pgp-border.singaren.net.sg [202.3.135.130]

18   *        *        *     Request timed out.

...

C:\>

1. In what city does UPenn's Magpi network connect to the Internet2's

Abilene network?

______________________________________________________________________

2. What US cities does the packet traverse in the Abilene network

(starting with the answer to 1 above)?

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

3. How long is the fiber link across the Pacific (assume light travels

at 200 meters/microsecond in fiber and that all of the delay is round

trip propagation delay)?

______________________________________________________________________

4. Who (probably) owns the fiber link across the Pacific Ocean?

______________________________________________________________________

8. When I executed the Windows command "netstat -an", on the small Windows

machine in my office (running Windows Server 2003), it produced the

following

c:/> netstat -an

Active Connections

Proto  Local Address          Foreign Address        State

TCP    0.0.0.0:7              0.0.0.0:0              LISTENING

TCP    0.0.0.0:9              0.0.0.0:0              LISTENING

TCP    0.0.0.0:13             0.0.0.0:0              LISTENING

TCP    0.0.0.0:17             0.0.0.0:0              LISTENING

TCP    0.0.0.0:19             0.0.0.0:0              LISTENING

TCP    0.0.0.0:42             0.0.0.0:0              LISTENING

TCP    0.0.0.0:53             0.0.0.0:0              LISTENING

TCP    0.0.0.0:80             0.0.0.0:0              LISTENING

TCP    0.0.0.0:135            0.0.0.0:0              LISTENING

TCP    0.0.0.0:445            0.0.0.0:0              LISTENING

TCP    0.0.0.0:515            0.0.0.0:0              LISTENING

TCP    0.0.0.0:548            0.0.0.0:0              LISTENING

TCP    0.0.0.0:1025           0.0.0.0:0              LISTENING

TCP    0.0.0.0:1026           0.0.0.0:0              LISTENING

TCP    0.0.0.0:1027           0.0.0.0:0              LISTENING

TCP    0.0.0.0:1030           0.0.0.0:0              LISTENING

TCP    0.0.0.0:1033           0.0.0.0:0              LISTENING

TCP    0.0.0.0:1037           0.0.0.0:0              LISTENING

TCP    0.0.0.0:1038           0.0.0.0:0              LISTENING

TCP    192.168.1.101:139      0.0.0.0:0              LISTENING

UDP    0.0.0.0:7              *:*

UDP    0.0.0.0:9              *:*

UDP    0.0.0.0:13             *:*

UDP    0.0.0.0:17             *:*

UDP    0.0.0.0:19             *:*

UDP    0.0.0.0:42             *:*

UDP    0.0.0.0:161            *:*

UDP    0.0.0.0:445            *:*

UDP    0.0.0.0:500            *:*

UDP    0.0.0.0:1029           *:*

UDP    0.0.0.0:1034           *:*

UDP    0.0.0.0:1036           *:*

UDP    0.0.0.0:1434           *:*

c:/>

What TCP and UDP services are being offered by this computer (Hint – only

Look at port numbers below 1024 - the others are probably dynamically

allocated). Note that each service is a potential "break-in" point.

________________    ________________    ________________    ________________

________________    ________________    ________________    ________________

________________    ________________    ________________    ________________

________________    ________________    ________________    ________________

________________    ________________    ________________    ________________

________________    ________________    ________________    ________________

9. Execute the command "netstat -a" (Windows) or netstat -lut (Unix) and list

any services (and break-in points) that your computer offers.

________________    ________________    ________________    ________________

________________    ________________    ________________    ________________

10. Enter the command "ipconfig -all (Windows) or ifconfig (Unix) and answer as

many of the following questions as possible.

a. IP address                                         ____.____.____.____

b. Subnet mask                                        ____.____.____.____

c. Default gateway or router                          ____.____.____.____

d. DNS nameservers                                    ____.____.____.____

e. DHCP server (if DHCP enabled)                      ____.____.____.____

f. Host name                        _____________________________________

g. Host domain name (DNS suffix)    _____________________________________