Introduction to Networks

1. Computers are typically connected by communication channels that provide

"speed of light" communication between computers attached to the channel.

Communication channels differ in:

a. The number of computers attached to the channel. A point-to-point

link connects two computers. Local area networks (LANs) may have tens

or hundred of computers attached to the same cable (like a party line

telephone).

b. The speed of the channel. For example:

1. modems - at up to 56,000 b/s (bits per second)

2. ADSL and cable modems at 256,000b/s to 1 Mb/s and more.

3. Leased communication lines such as a "T1" or "DS-1" line at 1.544 Mb/s

(millions of bits per second) or a T3" or "DS-3" line at 44 Mb/s.

4. Ethernet Local Area Networks (LAN's) at 10 or 100 Mb/s)

or gigabit Ethernet (1000 Mb/s = 1Gb/s).

5. SONET (Synchronous Optical NETwork) fiber links at 622 Mb/s to 10Gb/s.

c. The loss rate of the channel (percentage of frames lost or "damaged").

d. The distance covered and the propagation delay (light and electrical

signals travel at 200 to 300 meters per microsecond).

e. Cost, including fixed charge per minute or per month, or variable

charges based on bits transmitted and/or received.

2. Physical layer protocols (standards) specify the way that a computer sends a

single bit to another computer attached to the same communications channel.

3. Link layer protocols (standards) specify the way that a computer sends

a block of bits (called a frame) to another computer attached to the same

communications channel. Link layer protocols include:

a. Protocols for point-to-point links, such as SDLC, HDLC, LAP-B, and PPP.

b. Protocols for LAN's (Ethernet at various speeds).

c. Protocols for networks which simulate a set of point-to-point links

such as frame relay and ATM (Asynchronous Transfer Mode).

4. Instead of being directly connected, computers (called hosts, end systems,

or stations) my be "indirectly" connected to each using special computers

(called routers) that function like a post office. When a router receives a

packet, it forwards the packet to the "next" router to get the packet

closer to its ultimate destination (which might be across the city, across

the country, or around the world). Ten to twenty "hops" are common.

5. Network layer protocols specify the format of the packets (postcards) that

are forwarded by the routers (post office computers). The Internet Protocol

(the IP in TCP/IP) is one example. IPX (Novell) and X.25 are other examples.

6. Transport layer protocols (like Transmission Control Protocol - the TCP in

TCP/IP) make up for any deficiency in the service provided by the Network

layer. For example, IP provides "datagram" service (also called connection-

less service) which is like the post office delivering letters or postcards.

(Postcards may be lost or arrive out of order). TCP builds on the IP service

to provide guaranteed, in order, delivery of information (called a virtual

circuit or connection-oriented service).

7. Application layer protocols use the transport layer service to accomplish

something useful. For example:

a. File Transfer Protocol (FTP) transfers files between computers.

b. Simple Mail Transfer Protocol (SMTP) delivers E-Mail.

c. The Telnet protocol provides remote login service.

d. HyperText Transfer Protocol (HTTP) transfers World-Wide-Web (WWW) pages.

8. The Internet is a world-wide collection of millions of computers that can

exchange information in real time using the TCP/IP protocols. Most

academic computers at Temple University are connected to the Internet.

a. Computers on a floor within a building at Temple are typically connected

to an Ethernet "switcher" at 100 Mb/s using category 5 (Cat5) twisted

pair cable (high quality 8-wire telephone cable).

b. The bridges on each floor in a building are connected by fiber to an

IP router that might be located in the basement of the building.

c. The routers are connected by gigabit Ethernet to a "gigabit switches"

(very fast routers) in the Bell Building.

d. A router in the Bell Building (155.247.119.226) campus is connected to

Level3 (Temple's Internet service provider) which has an international

network of IP routers connected by at gigabit speeds. 155.247.119.226 is

also connected to MAGPI (the UPenn regional network) which is connected

to the Abilene 10Gb/s national network (Internet II).

e. Some of Level3's routers are located in special "interchange rooms"

called MAEs (Metropolitan Area Exchanges and NAPs (Network Access

Points) located around the country. The Level 3 IP routers in these

rooms exchange packets with routers from other Internet service

providers (e.g. Cogentco, Voicenet, AT&T, ANS, SPRINT, MCI, AT&T, etc.).

9. Counting systems.

base 10 - 345 (base 10) = 3**10*10 + 4*10 + 5*1 = 345 (base 10)

base 20 - 47 (base 20) = fourscore and seven = 4*20 + 7 = 87 (base 10)

base 12 - 2 gross, 3 dozen and 4 means 2*144 + 3+12 + 4*1 = 328

234 (base 12) = 328 (base 10)

base 60 - 2 hours, 3 minutes, 4 seconds = 2*3600 + 3*60 + 4*1 = 7384

seconds (base 10)

10. Binary counting system. Two symbols - 0 and 1. Each BInary digiT is called

a BIT (a "0" or a "1"). In binary, 1+1 = 10. To count in binary, just count

normally and throw out any number that contains a 2, 3, 4, 5, 6, 7, 8, or 9.

0, 1, 10, 11, 100, 101, 110, 111, 1000, 1001, 1010, 1011, 1100, ...

111000 (base 2) = 1*32 + 1*16 + 1*8 + 0*4 + 0*2 + 0*1 = 56 (base 10)

Computers use the binary system because it simplifies hardware.

11. Hexadecimal (base 16) counting system. 16 symbols (0 to 9 and A to F).

Count 0 1 2 3 4 5 6 7 8 9 A B C D E F 10 11 12 13 14 15 16 17 18 19 1A ...

Note that 1 + 9 = A, 1 + F = 10, 1 + 1F = 20, 1 + 1FF = 200, and

1 + FFF = 1000. AF3 (base 16) = 10*256 + 15*16 + 3*1 = 2803 (base 10)

12. Decimal, Binary, and Hexadecimal counting

decimal binary hex      decimal  binary  hex      decimal  binary  hex

0     0000   0          8      1000    8         16     10000   10

1     0001   1          9      1001    9         17     10001   11

2     0010   2         10      1010    A         18     10010   12

3     0011   3         11      1011    B         19     10011   13

4     0100   4         12      1100    C         20     10100   14

5     0101   5         13      1101    D         21     10101   15

6     0110   6         14      1110    E         22     10110   16

7     0111   7         15      1111    F         23     10111   17

13. IP Host Names and Addresses

a. Internet name - (shirley.cis.temple.edu) Unique worldwide. Used by

people but translated to IP address before communication takes place.

b. IP address - (129.32.1.64) Unique worldwide. 32 bits represented as 4

decimal numbers between 0 to 255.

decimal        129           32           01           64

hex             81           20           01           40

binary   1000 0001    0010 0000    0000 0001    0100 0000

In the address 129.32.01.64, the first 16 bits (129.32) indicate

that the computer is at Temple University and the last 16 bits

(1.64) identify a particular host (shirley) on Temple’s 129.32

network.

Temple has also been assigned the network number 155.147.xxx.yyy. This

network is "subnetted". In the address 155.147.xxx.yyy, 155.147

specifies Temple University, xxx specifies a particular Ethernet at

Temple, and yyy identifies a particular host on the Ethernet.

c. Ethernet address - (00:00:0f:00:7e:d9) Unique worldwide. 48 bits

represented as 12 hexadecimal bytes.

hex        0    0    0    0    0    f    0    0    7    e    d    9

binary  0000 0000 0000 0000 0000 1111 0000 0000 0111 1110 1110 1001

First 24 bits (00:00:0f) identify the manufacturer of the Ethernet board

(Next) who must assigns the last 24 bits (00:7e:d9) so that there are no

duplicates. Only used to send information between computers that are

connected to the same "wire" (the same Ethernet).

14. Packets and Protocols - Envelopes inside of envelopes.

I was in my office using my Unix computer (shirley) and I wanted a directory

listing from the computer cs.orst.edu at Oregon State. Shirley sent an

FTP packet inside a TCP packet inside an IP packet inside an Ethernet packet

to our router (then called prepnet.temple.edu). The prepnet router then

forwarded the packet toward Oregon. The relevant Ethernet and IP addresses

are:

shirley            prepnet            cs.orst.edu

IP address         129.32.1.64        129.32.16.1        128.193.32.1

Ethernet address   00:00:0f:00:7e:d9  00:00:93:e0:70:55

The protocol stack (Ethernet -> IP -> TCP -> FTP) is as follows:

Ethernet packet header

to        - 00:00:93:e0:70:55    (prepnet's Ethernet address)

from      - 00:00:0f:00:7e:d9    (shirley's Ethernet address)

type      - 0800                 (contains an IP packet)

Ethernet packet content - the IP packet below

IP packet header

to          - 128.193.32.1       (cs.orst.edu's IP address)

from        - 129.32.1.64        (shirley's IP address)

protocol    - 06                 (contains a TCP packet)

IP content - the TCP packet below

TCP packet header

to port      - 21           (All FTP servers listen on "port" 21)

from port    - 2590         (I was assigned port 2590 on shirley)

sequence #   - a2a4c24c     (Contains bytes starting with # a2a4c24c)

ack #        - 02916eca     (I expect byte # 02916eca from you next)

TCP packet content - the ftp packet below

FTP packet content - "LIST"     (ftp command requesting directory

listing from remote computer)

15. Example of an Ethernet frame containing an IP packet containing a TCP

message containing the FTP command "LIST". The last 4 lines are the

content of the 60 bytes that formed the Ethernet packet. The first 8 lines

are a “user friendly” version of the same information printed by the

Lanwatch software.

Receive time:  100.960   packet length: 60     received length: 60

Ethernet:   (00000f007ed9 -> Prote e07055)  type: IP(0x0800)

Internet:     129.32.1.64 -> 128.193.32.1    hl: 5   ver: 4   tos: 0

len: 46   id: 0x114d fragoff: 0   flags: 00 ttl: 30  prot: TCP(6)

xsum: 0x5b68

TCP:            2590 -> ftp(21)        seq: a2a4c24c  ack: 02916eca

win: 4096   hl: 5   xsum: 0x7cef  urg: 0      flags: <ACK><PUSH>

data (6/6): LIST

byte  hex contents of 16 bytes                             ASCII contents

0000: 00 00 93 e0 70 55 00 00 - 0f 00 7e d9 08 00 45 00  |    pU    ~   E |

0010: 00 2e 11 4d 00 00 1e 06 - 68 5b 81 20 01 40 80 c1  | . M    h[   @  |

0020: 20 01 0a 1e 00 15 a2 a4 - c2 4c 02 91 6e ca 50 18  |         L  n P |

0030: 10 00 ef 7c 00 00 4c 49 - 53 54 0d 0a              |   |  LIST  |

16. Brief description of several protocols

Ethernet - Transports IP or other network layer packets to computers on

local LAN. Broadcast address ff:ff:ff:ff:ff:ff delivers to all computers.

PPP - Point to Point Protocol - Transports IP or other network layer packets

over a point-to-point link such as a "T1" dedicated line or a modem

with a dial-up telephone connection.

ARP - Address Resolution Protocol - Use by computers on a LAN to "discover"

Ethernet address of a computer on the same LAN. Example - To Ethernet

address ff:ff:ff:ff:ff:ff, will the computer with IP address 129.32.1.3

please send me their Ethernet address.

IP - Internet Protocol - Based on the IP address, delivers packet to any of

the millions of computers on the Internet. Provides "datagram" service

(like a post card).

ICMP - Internet Control Message Protocol - Used by IP routers and hosts

to send error and information messages (ping and traceroute use ICMP).

UDP - User Datagram Protocol - Transport layer protocol that transfers

"datagrams" (like a postcard) between any two Internet computers.

Datagrams can be lost, duplicated or arrive out of order.

TCP - Transmission Control Protocol - Transport layer protocol that

provides reliable "virtual circuit" service (like a registered letter)

between any two computers attached to the internet.

DHCP – Dynamic Host Configuration Protocol – During the boot process, a

Client broadcasts a request for its IP address and other parameters.

DNS - Domain Name Service - Your computer sends a name to a name server

(e.g. shirley.cis.temple.edu) and the server returns the IP address

(e.g. 129.32.1.64).

HTTP - Hyper Text Transfer Protocol - Used to transfer WWW (World

Wide Web) pages.

FTP - File Transfer Protocol – To transfer files.

SMTP - Simple Mail Transfer Protocol - for electronic mail.

TELNET - For Remote Login. Also used by terminal servers.

17. What is inside of what (protocol layering)

application ----> DNS    DHCP         SMTP      FTP    TELNET    HTTP

|       |            |        |        |        |

|       |            |        |        |        |

|       |            |        |        |        |

+---+---+            +--------+----+---+--------+

|                              |

transport ------>     UDP                            TCP

|                              |

|             ICMP             |

|              |               |

+--------------+---------------+

|

network -------->                    IP<--------------+

|               |

ARP             |               |

|              |               |

+--------------+               |

|               |

link   -------->                  Ethernet          PPP

|               |

|               |

physical ------>                  Ethernet         modem

Protocol Descriptions

1. Ethernet frame

a. 48 bit (6 bytes or 12 hex digits) "TO" address (00:00:93:e0:70:55)

b. 48 bit (6 bytes or 12 hex digits) "From" address (00:00:0f:00:7e:d9)

c. 16 bit (2 bytes) "Type" field - 0806 for ARP, 0800 for IP

d. 48 to 1500 bytes of data

e. 32-bit (4 byte) CRC (check digits)

Ethernet hardware addresses (48 bits, expressed as 12 hexadecimal digits)

00-00-0F-??-??-??       NeXT

08-00-20-??-??-??       Sun

08-00-5A-??-??-??       IBM

FF-FF-FF-FF-FF-FF       broadcast address to all stations

2. APR protocol for Ethernet hardware and IP network address

(see http://www.rfc-editor.org/rfc/rfc826.txt)

1. 16-bit local area network type (0001 hex for Ethernet)

2. 16-bit network protocol type (0800 hex for IP)

3. 8-bit length of LAN address (06 for Ethernet)

4. 8-bit length of network protocol address (04 for IP)

5. 16-bit operation (0001 for request, 0002 for reply)

6. 48-bit source Ethernet address (e.g. 00000f007ed9)

7. 32-bit source IP address (e.g. 81200140 for 129.32.1.64)

8. 48-bit destination Ethernet address (e.g. 000000000000 in request,

000093e07055 in reply.

9. 32-bit destination IP address (e.g. 81201001 for 129.32.16.1)

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

| hardware addr type (1=ether)  | protocol addr type (0800=IP)  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|eth adr len(6) | IP adr len(4) | operation (1=request,2=reply) |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|     First 32 bits (4 bytes) of sender Ethernet address        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

| Last 2 bytes of sender Ether  |   First 2 bytes of sender IP  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|   Last 2 bytes of sender IP   |  First 2 bytes receiver Ether |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|     Last 32 bits (4 bytes) of receiver Ethernet address       |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                All 4 bytes of receiver IP  address            |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

3. IP Packet (see http://www.rfc-editor.org/rfc/rfc791.txt)

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|Version|  IHL  |Type of Service|          Total Length         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|         Identification        |Flags|      Fragment Offset    |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  Time to Live |    Protocol   |         Header Checksum       |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                       Source Address                          |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                    Destination Address                        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  TCP header, then your data ......                            |

|                                                               |

IHL - IP Header Length in 32 bit words (normally 5)

Total Length so you can find the end of the IP packet.

Identification and Fragment Offset for splitting long packets so

that the receiver can reassemble them.

Time to Live normally set to 64, 128, or 255 and decreased by 1 each time

the packet passes through an IP router. Discarded when TTL reaches 0.

Traceroute sends packet with time to live of 1, then sends same packet

with time to live of 2, then 3, etc. When IP router discards a packet

because time to live is zero, is returns a message to the sender.

Header checksum for error checking (easy to perform in software).

Protocol - The following table can be found in /etc/protocols

ip      0       IP      # internet protocol, pseudo protocol number

icmp    1       ICMP    # internet control message protocol

tcp     6       TCP     # transmission control protocol

udp     17      UDP     # user datagram protocol

ipv6    41      UDP     # IP version 6

IP Addresses are 32 bits long (e.g. 129.32.32.61)

Special IP addresses

0.0.0.0            this computer (use at bootstrap before IP assigned)

127.0.0.1          loopback address (computer connects to itself)

xxx.yyy.xxx.255    Broadcast address for the xxx.yyy.zzz net (can forward)

255.255.255.255    "This net" broadcast address (never forwarded)

IP addresses reserved for private networks (routers won't forward)

10.0.0.0     to  10.255.255.255

172.16.0.0   to  172.31.255.255

192.168.0.0  to  192.168.255.255

169.254.0.0  to  169.254.255.255

4. IPv6 Packet (see http://www.rfc-editor.org/rfc/rfc2460.txt)

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|Version| Traffic Class |           Flow Label                  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|         Payload Length        |  Next Header  |   Hop Limit   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                                                               |

+                                                               +

|                                                               |

+                         Source Address                        +

|                                                               |

+                                                               +

|                                                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                                                               |

+                                                               +

|                                                               |

+                      Destination Address                      +

|                                                               |

+                                                               +

|                                                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Proposed replacement for IP (IPv4) which increases the size of

addresses from 32 to 128 bits.

Version equals 6 (0110) instead of 4 (0100)

Traffic Class and Flow Label for implementing "quality of service)

Payload Length (instead of total length)

Next Header (instead of protocol).

Hop Limit (instead of time-to-live)

5. TCP Packet (see http://www.rfc-editor.org/rfc/rfc793.txt)

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|          Source Port          |       Destination Port        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                        Sequence Number                        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                    Acknowledgment Number                      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  Data |           |U|A|P|R|S|F|                               |

| Offset| Reserved  |R|C|S|S|Y|I|            Window             |

|       |           |G|K|H|T|N|N|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|           Checksum            |         Urgent Pointer        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|   your data ... next 500 octets                               |

|   ......                                                      |

Flags

URG - Like typing control C, slip data up to urgent data.

ACK - The acknowledgment number field is valid.

PSH - The sender requested that the data be sent immediately.

RST - Lets reset the connection and start over.

SYN - This is my first sequence number.

FIN - I would like to end this conversation.

6. UDP Packet3 (see http://www.rfc-editor.org/rfc/rfc768.txt)

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|          Source Port          |       Destination Port        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|            Length             |           Checksum            |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|   your data ... next 500 octets                               |

|   ......                                                      |

Source and Destination Ports

(see http://www.rfc-editor.org/rfc/rfc1700.txt)

echo             7/tcp          echo port

ftp             21/tcp          file transfer

telnet          23/tcp          remote login

smtp            25/tcp          E-mail

domain          53/udp          nameserver

finger          79/tcp          finger (find) compute user

http            80/tcp          WWW server

On UNIX systems, ports below 1024 are "protected" while higher numbered

ports are available just by asking. (Usually, the user program asks for

a port and UNIX returns an unused port number for you to use.)

7. ICMP Protocol (see http://www.rfc-editor.org/rfc/rfc792.txt)

A. ICMP packets are carried inside IP packets that have the 8-bit IP

protocol set to 1. The ICMP packet begins with an 8-bit type field that

specifies the ICMP message type. Types include:

0  Echo Reply                  response of system that receives and echo

3  Destination Unreachable

4  Source Quench               flow control

5  Redirect                    better router available on your subnet

8  Echo                        generated by ping

11  Time Exceeded               time to live went to 0 (so packet died)

12  Parameter Problem

13 and 14 Timestamp and Timestamp reply

15 and 16 Information Request and Information reply

B. Destination Unreachable Message (Type = 3)

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|   Type = 3    |     Code      |          Checksum             |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                             unused                            |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      Internet Header + 64 bits of Original Data Datagram      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Code  0 = net unreachable;

1 = host unreachable;

2 = protocol unreachable;

3 = port unreachable;

4 = fragmentation needed and DF set;

5 = source route failed.

C. Time Exceeded Message

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|   Type = 11   |     Code      |          Checksum             |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                             unused                            |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      Internet Header + 64 bits of Original Data Datagram      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Code  0 = time to live exceeded in transit;

1 = fragment reassembly time exceeded.

D. Echo or Echo Reply Message (ping request or reply)

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|     Type      |   Code = 0    |          Checksum             |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|           Identifier          |        Sequence Number        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|     Data ...

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ...

Type  8 for echo message;

0 for echo reply message.

Identifier - If code = 0, an identifier to aid in matching echoes

and replies, may be zero.

Many companies use firewalls to discard incoming ICMP packets, so ping and

traceroute may not work to external locations.

8. FTP - File Transfer Protocol (see http://www.rfc-editor.org/rfc/rfc959.txt)

A. Some FTP (client) COMMANDS

USER NAME (USER)               USER anonymous

PASSWORD (PASS)                PASS stafford@shirley.cis.temple.edu

CHANGE WORKING DIRECTORY (CWD) CWD pub

LOGOUT (QUIT)

DATA PORT (PORT)               PORT h1,h2,h3,h4,p1,p2

REPRESENTATION TYPE (TYPE)     TYPE I (for IMAGE) or TYPE A (for ASCII)

RETRIEVE (RETR)                RETR filename

STORE (STOR)                   STOR filename

DELETE (DELE)                  DELE filename

PRINT WORKING DIRECTORY (PWD)  PWD

LIST (LIST)                    LIST

SYSTEM (SYST)                  SYSTEM

STATUS (STAT)                  STATUS

HELP (HELP)                    HELP

NOOP (NOOP)                    NOOP

B. Some FTP (server) REPLIES. The reply codes fall into 5 classes:

1yz   Positive Preliminary reply (but wait for 2yz message)

2yz   Positive Completion reply

3yz   Positive Intermediate reply (command sequences, send next)

4yz   Transient Negative Completion reply

5yz   Permanent Negative Completion reply

125 Data connection already open; transfer starting.

150 File status okay; about to open data connection.

200 Command okay.

211 System status, or system help reply.

212 Directory status.

213 File status.

214 Help message.

215 NAME system type. Where NAME is an official system name.

220 Service ready for new user.

221 Service closing control connection. Logged out if appropriate.

225 Data connection open; no transfer in progress.

226 Closing data connection. Requested file action successful.

227 Entering Passive Mode (h1,h2,h3,h4,p1,p2).

230 User logged in, proceed.

250 Requested file action okay, completed.

331 User name okay, need password.

332 Need account for login.

350 Requested file action pending further information.

421 Service not available, closing control connection.

425 Can't open data connection.

426 Connection closed; transfer aborted.

450 Requested file action not taken. File unavailable (e.g., busy).

500 Syntax error, command unrecognized.

501 Syntax error in parameters or arguments.

502 Command not implemented.

503 Bad sequence of commands.

504 Command not implemented for that parameter.

530 Not logged in.

532 Need account for storing files.

550 Requested action not taken. File unavailable (e.g., not found)

553 Requested action not taken. File name not allowed.

9. Hyper Text Transfer Protocol

A. HyperText Transfer Protocol (HTTP) is a client/server application

protocol in which a client establishes a TCP session with a server

(which normally  listens on port 80) and than sends a single "request"

to the server. The  HTTP server responds by sending a single response to

the client and then  ending the TCP session. See

http://www.rfc-editor.org/rfc/rfc1945.txt (version 1.0)

http://www.rfc-editor.org/rfc/rfc2068.txt (version 1.1)

B. An HTTP request consists of a request line (GET /~stafford/ HTTP/1.0)

followed by optional header lines (e.g. “Host: unix.temple.edu”),

followed by a blank line, followed by an optional message body.

C. An HTTP response consists of a status line (e.g. HTTP/1.1 200 OK)

followed by various header lines (e.g. Content-length: 261) followed

by a blank line followed by an optional message body.

D. Instead of using a client such as Netscape or mosaic, it is possible to

use telnet as the client program. Using the telnet command to enter:

telnet astro.temple.edu 80

This connects you to the HTTP port (port 80) instead of the normal telnet

port (port 23). Next enter the following HTTP command:

GET /~stafford/short.html HTTP/1.0

and then press the Enter key twice (so that the GET command is followed

by a blank line). The HTTP server will respond by sending back the

following message (and then terminating the telnet session).

HTTP/1.1 200 OK

Date: Mon, 22 Dec 2003 20:45:13 GMT

Server: Apache/2.0.46 (Unix) PHP/4.3.2 mod_ssl/2.0.46 OpenSSL/0.9.7b

Last-Modified: Mon, 22 Dec 2003 20:38:59 GMT

ETag: "5e8a-108-3cf17ade81ec0"

Accept-Ranges: bytes

Content-Length: 264

Connection: close

Content-Type: text/html

<html>

<head><title>Short Page</title></head>

<body>

<h2>A Short HTML Page for Testing</h2>

This is just a short web page for testing.

My main page on astro.temple.edu/~stafford is

<a href="index.html">index.html</a>

</body>

</html>

E. If you would like to pretend to be an HTML 1.1 client, you need to

Include a “HOST:” header line (remember to press the Enter key twice).

GET /~stafford/short.html HTTP/1.1

HOST: astro.temple.edu                  (press Enter twice)

10. Domain Name Service (DNS) Protocol

A. The Domain Name System (DNS) implements a worldwide distributed database

in which clients use UDP port 53 to query name servers that implement the

database. See:

http://www.rfc-editor.org/rfc/rfc1034.txt (Concepts and Facilities)

http://www.rfc-editor.org/rfc/rfc1035.txt (Implementation and Spec.)

B. Types of queries and responses. Although RFC 1035 defines 16 classes of

queries, you are only likely to see 7 classes - SOA, NS, A, CNAME, HINFO,

MX and PTR.

1. A (internet Address). Translates an internet name

(e.g. astro.temple.edu) into an IP address (e.g. 155.247.166.104).

2. NS (Name Server). Specifies the IP address of a name server holding

the data for a section of the DNS database (e.g. names ending in

temple.edu). For example, 155.247.19.2 and 155.247.166.2 are the name

servers for "temple.edu".  Any query from anywhere in the world about

a name ending in temple.edu  will be answered, directly or indirectly,

from the duplicate database on these two computers.

5. CNAME (Canonical Name). Translates a nickname (e.g. ftp.temple.edu)

into a canonical (official) name (e.g. jane.ocis.temple.edu). Because

ftp.temple.edu is a CNAME record rather than an A record, Temple can

easily change its ftp server (e.g. from jane.ocis.temple.edu to

john.ocis.temple.edu).

6. SOA (Start of Authority). Specifies E-mail address of person

responsible for maintaining a section of the DNS database (e.g.

temple.edu or ibm.com) and parameters to use when caching information

from this section of the database.

12. PTR (PoinTeR). Translates an IP address (e.g. 155.247.166.180) into an

internet name (e.g. jane.ocis.temple.edu). Because the DNS database

assumes that the leftmost end of a name is the most significant, an

IP address like 155.247.166.180 is stored in the database as

"180.166.247.155.in-addr.arpa. (The in-addr.arpa part is historical).

13. HINFO (Host INFOrmation). Inputs an name (e.g. jane.ocis.temple.edu)

and returns a "comment" (e.g. CPU = 8th floor computer building

15. MX (Mail eXchange). Provides names of computers that will accept E-mail

for a particular name. For example, the MX record for temple.edu is

smtpauth.temple.edu so mail sent to stafford@temple.edu will result in

an SMTP connection to smtpauth.temple.edu.

C. Note that there is no computer with the name temple.edu. There are

actually 4 database records for temple.edu in the database on 155.247.19.2

and 155.247.166.2, but none of them is an "A" record.

temple.edu  SOA  dns@temple.edu, ...

temple.edu  NS   155.247.166.2

temple.edu  NS   155.247.19.2

temple.edu  MX   smtpauth.temple.edu

Because we didn't include a CNAME record like "CNAME temple.edu

www.temple.edu", in our temple database, the web address

"http://temple.edu" will fail. Because of the MX record, addresses like

stafford@temple.edu are translated to stafford@smtpauth.temple.edu

D. To play with the DNS database, enter the command "nslookup" on Unix,

Windows NT, 2000, or XP (but not 98 or ME) and enter ? to see the

commands. Try

"nslookup" -> "set type=mx" -> "temple.edu"

"nslookup" -> "server ns1.temple.edu" -> "ls temple.edu"

or (ls temple.edu > t.txt)

11. SMTP Protocol (see http://www.rfc-editor.org/rfc/rfc2821.txt)

Commands necessary to send a mail message

EHLO Domain                                    (new HELO command)

MAIL FROM:<reverse-path> <optional-parameters> (from address)

RCPT TO:<forward-path> <optional-parameters>   (several RECIPIENTs OK)

DATA                                           (ends with line ".")

QUIT

Some additional commands

HELO Domain                                     (old HELO command)

HELP                                            (to print these commands)

NOOP                                            (see if server is alive)

VRFY username                                   (is this a valid mailbox?)

Some Responses

211 System status, or system help reply

214 Help message

220 <domain> Service ready

221 <domain> Service closing transmission channel

250 Requested mail action okay, completed

251 User not local; will forward to <forward-path>

252 Cannot VRFY user, but will accept message and attempt delivery

354 Start mail input; end with <CRLF>.<CRLF>

I logged onto Temple’s unix system (astro.temple.edu) and issued the telnet

command “telnet smtpauth.temple.edu 25”. The commands that I entered start

with 4 capital letters (e.g. EHLO). The mail server’s responses start with

three digits (e.g. 220). (smtpauth only accepts mail from Temple computers.)

220 po-smtp2.temple.edu ESMTP Mirapoint 3.3.7-GR; Tue, 23 Dec 2003

09:50:57 -0500 (EST)

EHLO astro.temple.edu

250 po-smtp2.temple.edu Hello astro.temple.edu [155.247.166.104],

pleased to meet you

MAIL FROM:santa@santa.clause.org                       (from address)

250 santa@santa.clause.org... Sender ok

RCPT TO:stafford@temple.edu

250 stafford@temple.edu... Recipient ok                (to address)

DATA

354 Enter mail, end with "." on a line by itself

If you want anything for Christmas,                    (body of message)

you need to be nicer to your students.                 (body of message)

santa                                                  (body of message)

.                                                      (end of message)

250 AQM73206 Message accepted for delivery

QUIT

221 po-smtp2.temple.edu closing connection

Protocol Analysis using Lanwatch

The following is a log of an FTP session between shirley.cis.temple.edu

and cs.orst.edu. The characters I typed are underlined.

shirley> ftp cs.orst.edu

---------------

Connected to cs.orst.edu.

220 lynx FTP server (Version $Revision: 15.15 $

$Date: 89/08/31 10:33:40 $) ready.

Name (cs.orst.edu:stafford): anonymous

---------

331 Guest login ok, send ident as password.

Password: stafford@shirley.temple.edu

---------------------------

230 Guest login ok, access restrictions apply.

ftp> dir

---

200 PORT command okay.

150 Opening data connection for /bin/ls -l (129.32.1.64,2595) (0 bytes).

total 6

dr-xr-xr-x   2 root     root        1024 Oct 15  1990 bin

dr-xr-xr-x   2 root     root        1024 Jun 13 16:22 etc

drwxr-xr-x  24 root     sys         1024 Sep 10 19:55 pub

226 Transfer complete.

186 bytes received in 0.05 seconds (3.74 Kbytes/s)

ftp> bye

---

221 Goodbye.

shirley>

The following pages contain a trace of the 42 packets generated on Temple's

Ethernet by the session above. The packets were captured using LANWatch

network analyzer software from FTP software. The computers involved are:

Internet Name            IP Address    Ethernet Address   Function

shirley.ocis.temple.edu  129.32.1.64   00:00:0f:00:7e:d9  My Next

prepnet.temple.edu       129.32.16.1   00:00:93:e0:70:55  Temple router

charon.psc.edu           128.182.65.6                     Temple nameserver

cs.orst.edu              129.193.32.1                     Oregon CS computer

A total of 42 Ethernet packets were captured. The first 2 Ethernet packets

contain ARP packets that shirley uses to find out the Ethernet address of

prepnet.temple.edu. The remaining 40 Ethernet packets contain IP packets.

Shirley knows that the Temple router, prepnet.temple.edu, has an IP address

of 129.32.16.1. However, shirley does not know the routers Ethernet address

(00:00:93:e0:70:55). Packets 1 and 2 are "arp" protocol packets that shirley

uses to find out the Ethernet address.

Packet 1 - shirley to everyone - will 129.32.16.1 send me their Ethernet addr.

Receive time:   81.901   packet length: 60     received length: 60

Ethernet:   (00000f007ed9 ->       *     )  type: ARP(0x0806)

ARP: rqst(1):  hdwr: 1  prot: IP(0x0800)  hln: 6  pln: 4

Prot. Addrs:    129.32.1.64 -> 129.32.16.1

Hrdw. Addrs: 00000f007ed9 ->       *

0000: ff ff ff ff ff ff 00 00 - 0f 00 7e d9 08 06 00 01      |          ~     |

0010: 08 00 06 04 00 01 00 00 - 0f 00 7e d9 81 20 01 40      |          ~     |

0020: ff ff ff ff ff ff 81 20 - 10 01 02 01 00 00 00 02      |                |

0030: 00 00 c0 05 92 00 00 00 - 00 00 00 00                  |            |

Packet 2 - prepnet to shirley - my ethernet address is 00:00:93:e0:70:55

Receive time:   81.902   packet length: 60     received length: 60

Ethernet:   (Prote e07055 -> 00000f007ed9)  type: ARP(0x0806)

ARP: rply(2):  hdwr: 1  prot: IP(0x0800)  hln: 6  pln: 4

Prot. Addrs:    129.32.16.1 -> 129.32.1.64

Hrdw. Addrs: Prote e07055 -> 00000f007ed9

0000: 00 00 0f 00 7e d9 00 00 - 93 e0 70 55 08 06 00 01      |    ~     pU    |

0010: 08 00 06 04 00 02 00 00 - 93 e0 70 55 81 20 10 01      |          pU    |

0020: 00 00 0f 00 7e d9 81 20 - 01 40 02 01 00 00 00 02      |    ~    @      |

0030: 00 00 c0 05 92 00 00 00 - 00 00 00 00                  |            |

In order to send packets to cs.orst.edu, shirley needs to know its IP address.

To answer such questions, shirley uses Temple’s nameservers (which at the time

were comvax.ocis.temple.edu at address 129.32.1.2 and charon.psc.edu at address 128.182.65.6). However, comvax happened to be "down" at the time, so shirley

asked used charon.psc.edu (which happens to be located in Pittsburgh. Shirley

first assumes that "cs.orst.edu" is at temple and, in packet 3, asks charon

about the name "cs.orst.edu.temple.edu". In packet 4, Shirley receives a

negative response from charon. In packet 5. shirley asks about the name

"cs.orst.edu" and receives the answer (and a lot of additional information) in

packet 6.

Packet 3 - shirley to charon - what is the IP address of cs.orst.edu.temple.edu

Receive time:   81.903   packet length: 82     received length: 82

Ethernet:   (00000f007ed9 -> Prote e07055)  type: IP(0x0800)

Internet:     129.32.1.64 -> 128.182.65.6    hl: 5   ver: 4   tos: 0

len: 68   id: 0x1139 fragoff: 0   flags: 00 ttl: 30  prot: UDP(17)

xsum: 0x5447

UDP:  4382 -> domain(53)  len: 48  xsum: 0xcc98

Domain: opcode: Query (0) Flags: <DORECURSE> (0100)

Queries: 1, answers: 0, name servers: 0, authoritative answers: 0

Query Number 0: Name: cs.orst.edu.temple.edu Type: Address (1)

0000: 00 00 93 e0 70 55 00 00 - 0f 00 7e d9 08 00 45 00      |    pU    ~   E |

0010: 00 44 11 39 00 00 1e 11 - 47 54 81 20 01 40 80 b6      | D 9    GT   @  |

0020: 41 06 11 1e 00 35 00 30 - cc 98 00 a4 01 00 00 01      |     5 0        |

0030: 00 00 00 00 00 00 02 63 - 73 04 6f 72 73 74 03 65      |       cs orst e|

0040: 64 75 06 74 65 6d 70 6c - 65 03 65 64 75 00 00 01      |du temple edu   |

0050: 00 01                                                  |  |

Packet 4 - charon to shirley - no such computer

Receive time:   81.953   packet length: 165    received length: 165

Ethernet:   (Prote e07055 -> 00000f007ed9)  type: IP(0x0800)

Internet:    128.182.65.6 -> 129.32.1.64     hl: 5   ver: 4   tos: 0

len: 151  id: 0x2ed9 fragoff: 0   flags: 00 ttl: 23  prot: UDP(17)

xsum: 0x6130

UDP:  domain(53) -> 4382  len: 131  xsum: 0xd2ca

Domain: opcode: Query (0) Response: Name error (3)

Flags: <RESPONSE><AUTHORITATIVE><DORECURSE><CANRECURSE> (8580)

Queries: 1, answers: 0, name servers: 1, authoritative answers: 0

Query Number 0: Name: cs.orst.edu.temple.edu Type: Address (1)

Class: Internet address (1)

0000: 00 00 0f 00 7e d9 00 00 - 93 e0 70 55 08 00 45 00      |    ~     pU  E |

0010: 00 97 2e d9 00 00 17 11 - 30 61 80 b6 41 06 81 20      |  .     0a  A   |

0020: 01 40 00 35 11 1e 00 83 - d2 ca 00 a4 85 83 00 01      | @ 5            |

0030: 00 00 00 01 00 00 02 63 - 73 04 6f 72 73 74 03 65      |       cs orst e|

0040: 64 75 06 74 65 6d 70 6c - 65 03 65 64 75 00 00 01      |du temple edu   |

0050: 00 01 06 74 65 6d 70 6c - 65 03 45 44 55 00 00 06      |   temple EDU   |

0060: 00 01 00 00 0e 10 00 3d - 06 63 6f 6d 76 61 78 04      |       = comvax |

0070: 6f 63 69 73 06 74 65 6d - 70 6c 65 03 65 64 75 00      |ocis temple edu |

0080: 06 73 77 61 7a 75 6b 03 - 66 61 63 03 63 69 73 c0      | swazuk fac cis |

0090: 4a 00 00 00 0d 00 00 0e - 10 00 00 01 2c 00 0a fc      |J           ,   |

00a0: 80 00 00 0e 10                                         |     |

Packet 5 - shirley to charon - "what is the IP address of cs.orst.edu"

Receive time:   81.960   packet length: 71     received length: 71

Ethernet:   (00000f007ed9 -> Prote e07055)  type: IP(0x0800)

Internet:     129.32.1.64 -> 128.182.65.6    hl: 5   ver: 4   tos: 0

len: 57   id: 0x113b fragoff: 0   flags: 00 ttl: 30  prot: UDP(17)

xsum: 0x5d47

UDP:  4383 -> domain(53)  len: 37  xsum: 0xe664

Domain: opcode: Query (0) Flags: <DORECURSE> (0100)

Queries: 1, answers: 0, name servers: 0, authoritative answers: 0

Query Number 0: Name: cs.orst.edu Type: Address (1)

0000: 00 00 93 e0 70 55 00 00 - 0f 00 7e d9 08 00 45 00      |    pU    ~Y  E |

0010: 00 39 11 3b 00 00 1e 11 - 47 5d 81 20 01 40 80 b6      | 9 ;    G]   @  |

0020: 41 06 11 1f 00 35 00 25 - e6 64 00 a5 01 00 00 01      |A    5 % d      |

0030: 00 00 00 00 00 00 02 63 - 73 04 6f 72 73 74 03 65      |       cs orst e|

0040: 64 75 00 00 01 00 01                                   |du     |

Packet 6 - charon to shirley - 128.193.32.1 is the address of cs.orst.edu

Receive time:   82.014   packet length: 369    received length: 369

Ethernet:   (Prote e07055 -> 00000f007ed9)  type: IP(0x0800)

Internet:    128.182.65.6 -> 129.32.1.64     hl: 5   ver: 4   tos: 0

len: 355  id: 0x2eda fragoff: 0   flags: 00 ttl: 23  prot: UDP(17)

xsum: 0x942f

UDP:  domain(53) -> 4383  len: 335  xsum: 0xef3d

Domain: opcode: Query (0) Response: No error (0)

Flags: <RESPONSE><DORECURSE><CANRECURSE> (8180)

Queries: 1, answers: 1, name servers: 7, authoritative answers: 8

Query Number 0: Name: cs.orst.edu Type: Address (1)

Class: Internet address (1)

Answer Number 0: Name: cs.orst.edu Type: Address (1)

Class: Internet address (1) TTL: 170336 Data (4 bytes):

IP address: 128.193.32.1

0000: 00 00 0f 00 7e d9 00 00 - 93 e0 70 55 08 00 45 00      |    ~     pU  E |

0010: 01 63 2e da 00 00 17 11 - 2f 94 80 b6 41 06 81 20      | c.     /   A   |

0020: 01 40 00 35 11 1f 01 4f - ef 3d 00 a5 81 80 00 01      | @ 5   O =      |

0030: 00 01 00 07 00 08 02 63 - 73 04 6f 72 73 74 03 65      |       cs orst e|

0040: 64 75 00 00 01 00 01 c0 - 0c 00 01 00 01 00 02 99      |du              |

0050: 60 00 04 80 c1 20 01 02 - 63 73 04 6f 72 73 74 03      |`       cs.orst |

0060: 45 44 55 00 00 02 00 01 - 00 00 25 51 00 0a 02 43      |EDU       %Q   C|

0070: 53 04 4f 52 53 54 c0 35 - c0 2d 00 02 00 01 00 00      |S ORST 5 -      |

0080: 25 51 00 0e 07 62 65 61 - 73 6c 65 79 03 55 43 53      |%Q   beasley UCS|

0090: c0 47 c0 2d 00 02 00 01 - 00 00 25 51 00 06 03 45      | G -      %Q   E|

00a0: 43 45 c0 47 c0 2d 00 02 - 00 01 00 00 25 51 00 06      |CE G -      %Q  |

00b0: 03 4f 43 45 c0 47 c0 2d - 00 02 00 01 00 00 25 51      | OCE G -      %Q|

00c0: 00 0e 04 6e 6e 73 63 03 - 4e 53 46 03 4e 45 54 00      |   nnsc NSF NET |

00d0: c0 2d 00 02 00 01 00 00 - 25 51 00 02 c0 62 c0 2d      | -      %Q   b -|

00e0: 00 02 00 01 00 00 25 51 - 00 07 04 6d 69 73 74 c0      |      %Q   mist |

00f0: 44 c0 44 00 01 00 01 00 - 02 99 60 00 04 80 c1 20      |D D       `     |

0100: 01 c0 5a 00 01 00 01 00 - 00 91 1e 00 04 80 c1 80      |  Z             |

0110: 03 c0 74 00 01 00 01 00 - 02 99 60 00 04 80 c1 30      |  t       `    0|

0120: 01 c0 86 00 01 00 01 00 - 02 99 60 00 04 80 c1 40      |          `    @|

0130: 01 c0 98 00 01 00 01 00 - 02 a0 ec 00 04 c0 1f 67      |               g|

0140: 06 c0 98 00 01 00 01 00 - 02 a0 ec 00 04 80 59 01      |          l     |

0150: b2 c0 62 00 01 00 01 00 - 02 99 60 00 04 80 c1 80      |  b       `     |

0160: 03 c0 c0 00 01 00 01 00 - 00 83 59 00 04 80 c1 20      |          Y     |

0170: 02                                                     | |

Shirley now initiates a TCP session with cs.orst.edu.  The IP protocol only

provides "datagram" service which means that packets may be lost, duplicated,

or arrive out of order. (Think of a letter with an unreliable Post Office.)

Building on IP, TCP provides "virtual circuit" service which guarantees

delivery of a stream of bytes in the correct order without loss or duplication.

(Think of a registered letter.) A TCP session begins with a "three-way

handshake". In packet 7, shirley sends the sequence number it will use for

counting bytes (SYN and a2a4c201). In packet 8, cs.orst.edu "acks" the packet

and sends its own sequence number (ACK, SYN, and 02916e00). In packet 9, Shirley

acks packet 8 (ACK).

Packet 7 - shirley to cs - I am calling the first data byte I send byte a2a4c202

Receive time:   82.017   packet length: 60     received length: 60

Ethernet:   (00000f007ed9 -> Prote e07055)  type: IP(0x0800)

Internet:     129.32.1.64 -> 128.193.32.1    hl: 5   ver: 4   tos: 0

len: 40   id: 0x113d fragoff: 0   flags: 00 ttl: 30  prot: TCP(6)

xsum: 0x7168

TCP:            2590 -> ftp(21)        seq: a2a4c201  ack:   ----

win: 4096   hl: 5   xsum: 0xe70d  urg: 0      flags: <SYN>

0000: 00 00 93 e0 70 55 00 00 - 0f 00 7e d9 08 00 45 00      |    pU    ~Y  E |

0010: 00 28 11 3d 00 00 1e 06 - 68 71 81 20 01 40 80 c1      | ( =    hq   @  |

0020: 20 01 0a 1e 00 15 a2 a4 - c2 01 00 00 00 00 50 02      |              P |

0030: 10 00 0d e7 00 00 02 63 - 73 04 6f 72                  |       cs or|

Packet 8 - cs to charon - OK, and my first byte will be byte number 02916e01

Receive time:   82.370   packet length: 60     received length: 60

Ethernet:   (Prote e07055 -> 00000f007ed9)  type: IP(0x0800)

Internet:    128.193.32.1 -> 129.32.1.64     hl: 5   ver: 4   tos: 0

len: 40   id: 0x8adc fragoff: 0   flags: 00 ttl: 13  prot: TCP(6)

xsum: 0xd1ff

TCP:         ftp(21) -> 2590           seq: 02916e00  ack: a2a4c202

win: 0      hl: 5   xsum: 0x44ad  urg: 0      flags: <ACK><SYN>

0000: 00 00 0f 00 7e d9 00 00 - 93 e0 70 55 08 00 45 00      |    ~     pU  E |

0010: 00 28 8a dc 00 00 0d 06 - ff d1 80 c1 20 01 81 20      | (              |

0020: 01 40 00 15 0a 1e 02 91 - 6e 00 a2 a4 c2 02 50 12      | @      n     P |

0030: 00 00 ad 44 00 00 40 60 - 00 00 00 00                  |   D  @`    |

Packet 9 - shirley to cs - ok

Receive time:   82.371   packet length: 60     received length: 60

Ethernet:   (00000f007ed9 -> Prote e07055)  type: IP(0x0800)

Internet:     129.32.1.64 -> 128.193.32.1    hl: 5   ver: 4   tos: 0

len: 40   id: 0x113e fragoff: 0   flags: 00 ttl: 30  prot: TCP(6)

xsum: 0x7068

TCP:            2590 -> ftp(21)        seq: a2a4c202  ack: 02916e01

win: 4096   hl: 5   xsum: 0x459d  urg: 0      flags: <ACK>

0000: 00 00 93 e0 70 55 00 00 - 0f 00 7e d9 08 00 45 00      |    pU    ~   E |

0010: 00 28 11 3e 00 00 1e 06 - 68 70 81 20 01 40 80 c1      | ( >    hp   @  |

0020: 20 01 0a 1e 00 15 a2 a4 - c2 02 02 91 6e 01 50 10      |            n P |

0030: 10 00 9d 45 00 00 40 60 - 00 00 00 00                  |   E  @`    |

Packet 10 - cs to shirley - ok

Receive time:   82.620   packet length: 60     received length: 60

Ethernet:   (Prote e07055 -> 00000f007ed9)  type: IP(0x0800)

Internet:    128.193.32.1 -> 129.32.1.64     hl: 5   ver: 4   tos: 0

len: 40   id: 0x8ae1 fragoff: 0   flags: 00 ttl: 13  prot: TCP(6)

xsum: 0xccff

TCP:         ftp(21) -> 2590           seq: 02916e01  ack: a2a4c202

win: 4096   hl: 5   xsum: 0x459d  urg: 0      flags: <ACK>

0000: 00 00 0f 00 7e d9 00 00 - 93 e0 70 55 08 00 45 00      |    ~     pU  E |

0010: 00 28 8a e1 00 00 0d 06 - ff cc 80 c1 20 01 81 20      | (              |

0020: 01 40 00 15 0a 1e 02 91 - 6e 01 a2 a4 c2 02 50 10      | @      n     P |

0030: 10 00 9d 45 00 00 40 60 - 00 00 00 00                  |   E  @`    |

Shirley initiated its TCP session with port 21 on cs.orst.edu. By convention,

the FTP server software listens for incoming connections on port 21. The

FTP server initiates an FTP session with shirley.

Packet 11 - cs to shirley - "220 lynx FTP server ..."

Receive time:   82.798   packet length: 138    received length: 138

Ethernet:   (Prote e07055 -> 00000f007ed9)  type: IP(0x0800)

Internet:    128.193.32.1 -> 129.32.1.64     hl: 5   ver: 4   tos: 0

len: 124  id: 0x8ae8 fragoff: 0   flags: 00 ttl: 13  prot: TCP(6)

xsum: 0x71ff

TCP:         ftp(21) -> 2590           seq: 02916e01  ack: a2a4c202

win: 4096   hl: 5   xsum: 0x7702  urg: 0      flags: <ACK><PUSH>

data (60/84): 220 lynx FTP server (Version $Revision: 15.15 $ $Date: 89/08

0000: 00 00 0f 00 7e d9 00 00 - 93 e0 70 55 08 00 45 00      |    ~     pU  E |

0010: 00 7c 8a e8 00 00 0d 06 - ff 71 80 c1 20 01 81 20      | |       q      |

0020: 01 40 00 15 0a 1e 02 91 - 6e 01 a2 a4 c2 02 50 18      | @      n     P |

0030: 10 00 02 77 00 00 32 32 - 30 20 6c 79 6e 78 20 46      |   w  220 lynx F|

0040: 54 50 20 73 65 72 76 65 - 72 20 28 56 65 72 73 69      |TP server (Versi|

0050: 6f 6e 20 24 52 65 76 69 - 73 69 6f 6e 3a 20 31 35      |on $Revision: 15|

0060: 2e 31 35 20 24 20 24 44 - 61 74 65 3a 20 38 39 2f      |.15 $ $Date: 89/|

0070: 30 38 2f 33 31 20 31 30 - 3a 33 33 3a 34 30 20 24      |08/31 10:33:40 $|

0080: 29 20 72 65 61 64 79 2e - 0d 0a                        |) ready.  |

Packet 12 - shirley to cs - ok

Receive time:   82.942   packet length: 60     received length: 60

Ethernet:   (00000f007ed9 -> Prote e07055)  type: IP(0x0800)

Internet:     129.32.1.64 -> 128.193.32.1    hl: 5   ver: 4   tos: 0

len: 40   id: 0x1141 fragoff: 0   flags: 00 ttl: 30  prot: TCP(6)

xsum: 0x6d68

TCP:            2590 -> ftp(21)        seq: a2a4c202  ack: 02916e55

win: 4096   hl: 5   xsum: 0xf19c  urg: 0      flags: <ACK>

0000: 00 00 93 e0 70 55 00 00 - 0f 00 7e d9 08 00 45 00      |    pU    ~Y  E |

0010: 00 28 11 41 00 00 1e 06 - 68 6d 81 20 01 40 80 c1      | ( A    hm   @  |

0020: 20 01 0a 1e 00 15 a2 a4 - c2 02 02 91 6e 55 50 10      |            nUP |

0030: 10 00 9c f1 00 00 32 32 - 30 20 6c 79                  |      220 ly|

Packet 13 - shirley to cs - "USER anonymous"

Receive time:   88.172   packet length: 70     received length: 70

Ethernet:   (00000f007ed9 -> Prote e07055)  type: IP(0x0800)

Internet:     129.32.1.64 -> 128.193.32.1    hl: 5   ver: 4   tos: 0

len: 56   id: 0x1143 fragoff: 0   flags: 00 ttl: 30  prot: TCP(6)

xsum: 0x5b68

TCP:            2590 -> ftp(21)        seq: a2a4c202  ack: 02916e55

win: 4096   hl: 5   xsum: 0xfd14  urg: 0      flags: <ACK><PUSH>

data (16/16): USER anonymous

0000: 00 00 93 e0 70 55 00 00 - 0f 00 7e d9 08 00 45 00      |    pU    ~   E |

0010: 00 38 11 43 00 00 1e 06 - 68 5b 81 20 01 40 80 c1      | 8 C    h[   @  |

0020: 20 01 0a 1e 00 15 a2 a4 - c2 02 02 91 6e 55 50 18      |            nUP |

0030: 10 00 14 fd 00 00 55 53 - 45 52 20 61 6e 6f 6e 79      |      USER anony|

0040: 6d 6f 75 73 0d 0a                                      |mous  |

Packet 14 - cs to shirley - "331 Guest login ok, ..."

Receive time:   88.476   packet length: 99     received length: 99

Ethernet:   (Prote e07055 -> 00000f007ed9)  type: IP(0x0800)

Internet:    128.193.32.1 -> 129.32.1.64     hl: 5   ver: 4   tos: 0

len: 85   id: 0x8b46 fragoff: 0   flags: 00 ttl: 13  prot: TCP(6)

xsum: 0x3aff

TCP:         ftp(21) -> 2590           seq: 02916e55  ack: a2a4c212

win: 4096   hl: 5   xsum: 0x86cb  urg: 0      flags: <ACK><PUSH>

data (45/45): 331 Guest login ok, send ident as password.

0000: 00 00 0f 00 7e d9 00 00 - 93 e0 70 55 08 00 45 00      |    ~     pU  E |

0010: 00 55 8b 46 00 00 0d 06 - ff 3a 80 c1 20 01 81 20      | U F     :      |

0020: 01 40 00 15 0a 1e 02 91 - 6e 55 a2 a4 c2 12 50 18      | @      nU    P |

0030: 10 00 cb 86 00 00 33 33 - 31 20 47 75 65 73 74 20      |      331 Guest |

0040: 6c 6f 67 69 6e 20 6f 6b - 2c 20 73 65 6e 64 20 69      |login ok, send i|

0050: 64 65 6e 74 20 61 73 20 - 70 61 73 73 77 6f 72 64      |dent as password|

0060: 2e 0d 0a                                               |.  |

Packet 15 - shirley to cs - ok

Receive time:   88.581   packet length: 60     received length: 60

Ethernet:   (00000f007ed9 -> Prote e07055)  type: IP(0x0800)

Internet:     129.32.1.64 -> 128.193.32.1    hl: 5   ver: 4   tos: 0

len: 40   id: 0x1144 fragoff: 0   flags: 00 ttl: 30  prot: TCP(6)

xsum: 0x6a68

TCP:            2590 -> ftp(21)        seq: a2a4c212  ack: 02916e82

win: 4096   hl: 5   xsum: 0xb49c  urg: 0      flags: <ACK>

0000: 00 00 93 e0 70 55 00 00 - 0f 00 7e d9 08 00 45 00      |    pU    ~Y  E |

0010: 00 28 11 44 00 00 1e 06 - 68 6a 81 20 01 40 80 c1      | ( D    hj   @  |

0020: 20 01 0a 1e 00 15 a2 a4 - c2 12 02 91 6e 82 50 10      |            n P |

0030: 10 00 9c b4 00 00 33 33 - 31 20 47 75                  |      331 Gu|

Packet 16 - shirley to cs - "PASS stafford@shirley.temple.edu"

Receive time:   96.944   packet length: 88     received length: 88

Ethernet:   (00000f007ed9 -> Prote e07055)  type: IP(0x0800)

Internet:     129.32.1.64 -> 128.193.32.1    hl: 5   ver: 4   tos: 0

len: 74   id: 0x1147 fragoff: 0   flags: 00 ttl: 30  prot: TCP(6)

xsum: 0x4568

TCP:            2590 -> ftp(21)        seq: a2a4c212  ack: 02916e82

win: 4096   hl: 5   xsum: 0x7979  urg: 0      flags: <ACK><PUSH>

data (34/34): PASS stafford@shirley.temple.edu

0000: 00 00 93 e0 70 55 00 00 - 0f 00 7e d9 08 00 45 00      |    pU    ~   E |

0010: 00 4a 11 47 00 00 1e 06 - 68 45 81 20 01 40 80 c1      | J G    hE   @  |

0020: 20 01 0a 1e 00 15 a2 a4 - c2 12 02 91 6e 82 50 18      |            n P |

0030: 10 00 79 79 00 00 50 41 - 53 53 20 73 74 61 66 66      |  yy  PASS staff|

0040: 6f 72 64 40 73 68 69 72 - 6c 65 79 2e 74 65 6d 70      |ord@shirley.temp|

0050: 6c 65 2e 65 64 75 0d 0a                                |le.edu  |

Packet 17 - cs to shirley - ok

Receive time:   97.210   packet length: 60     received length: 60

Ethernet:   (Prote e07055 -> 00000f007ed9)  type: IP(0x0800)

Internet:    128.193.32.1 -> 129.32.1.64     hl: 5   ver: 4   tos: 0

len: 40   id: 0x8b72 fragoff: 0   flags: 00 ttl: 13  prot: TCP(6)

xsum: 0x3bff

TCP:         ftp(21) -> 2590           seq: 02916e82  ack: a2a4c234

win: 4096   hl: 5   xsum: 0x929c  urg: 0      flags: <ACK>

0000: 00 00 0f 00 7e d9 00 00 - 93 e0 70 55 08 00 45 00      |    ~     pU  E |

0010: 00 28 8b 72 00 00 0d 06 - ff 3b 80 c1 20 01 81 20      | ( r     ;      |

0020: 01 40 00 15 0a 1e 02 91 - 6e 82 a2 a4 c2 34 50 10      | @      n    4P |

0030: 10 00 9c 92 00 00 40 60 - 1c 1d 1e 1f                  |      @`    |

Packet 18 - cs to shirley - "230 Guest login ok, ..."

Receive time:   97.213   packet length: 102    received length: 102

Ethernet:   (Prote e07055 -> 00000f007ed9)  type: IP(0x0800)

Internet:    128.193.32.1 -> 129.32.1.64     hl: 5   ver: 4   tos: 0

len: 88   id: 0x8b73 fragoff: 0   flags: 00 ttl: 13  prot: TCP(6)

xsum: 0x0aff

TCP:         ftp(21) -> 2590           seq: 02916e82  ack: a2a4c234

win: 4096   hl: 5   xsum: 0x73f0  urg: 0      flags: <ACK><PUSH>

data (48/48): 230 Guest login ok, access restrictions apply.

0000: 00 00 0f 00 7e d9 00 00 - 93 e0 70 55 08 00 45 00      |    ~     pU  E |

0010: 00 58 8b 73 00 00 0d 06 - ff 0a 80 c1 20 01 81 20      | X s            |

0020: 01 40 00 15 0a 1e 02 91 - 6e 82 a2 a4 c2 34 50 18      | @      n    4P |

0030: 10 00 f0 73 00 00 32 33 - 30 20 47 75 65 73 74 20      |   s  230 Guest |

0040: 6c 6f 67 69 6e 20 6f 6b - 2c 20 61 63 63 65 73 73      |login ok, access|

0050: 20 72 65 73 74 72 69 63 - 74 69 6f 6e 73 20 61 70      | restrictions ap|

0060: 70 6c 79 2e 0d 0a                                      |ply.  |

Packet 19 - shirley to cs - ok

Receive time:   97.389   packet length: 60     received length: 60

Ethernet:   (00000f007ed9 -> Prote e07055)  type: IP(0x0800)

Internet:     129.32.1.64 -> 128.193.32.1    hl: 5   ver: 4   tos: 0

len: 40   id: 0x1148 fragoff: 0   flags: 00 ttl: 30  prot: TCP(6)

xsum: 0x6668

TCP:            2590 -> ftp(21)        seq: a2a4c234  ack: 02916eb2

win: 4096   hl: 5   xsum: 0x629c  urg: 0      flags: <ACK>

0000: 00 00 93 e0 70 55 00 00 - 0f 00 7e d9 08 00 45 00      |    pU    ~   E |

0010: 00 28 11 48 00 00 1e 06 - 68 66 81 20 01 40 80 c1      | ( H    hf   @  |

0020: 20 01 0a 1e 00 15 a2 a4 - c2 34 02 91 6e b2 50 10      |         4  n P |

0030: 10 00 9c 62 00 00 32 33 - 30 20 47 75                  |   b  230 Gu|

The conversation being monitored is between port 2590 on 129.32.1.64 (shirley)

and port 21 (the ftp server) on 128.193.32.1 (cs.orst.edu). At any point

in time, these 4 numbers uniquely identify a conversation (like a pair of

telephone numbers). This conversation is called the "command channel". The

FTP protocol specifies that a second conversation should be used to transmit

data (the data channel). In this case, the data channel is between port 2591

on 129.32.1.64 (shirley) and port 20 (the ftp data port) on 128.193.32.1

(cs.orst.edu).

Packet 20 - shirley to cs - use port 10*256+31=2591 when you send data

Receive time:   99.405   packet length: 78     received length: 78

Ethernet:   (00000f007ed9 -> Prote e07055)  type: IP(0x0800)

Internet:     129.32.1.64 -> 128.193.32.1    hl: 5   ver: 4   tos: 0

len: 64   id: 0x114b fragoff: 0   flags: 00 ttl: 30  prot: TCP(6)

xsum: 0x4b68

TCP:            2590 -> ftp(21)        seq: a2a4c234  ack: 02916eb2

win: 4096   hl: 5   xsum: 0xdc44  urg: 0      flags: <ACK><PUSH>

data (24/24): PORT 129,32,1,64,10,31

0000: 00 00 93 e0 70 55 00 00 - 0f 00 7e d9 08 00 45 00      |    pU    ~   E |

0010: 00 40 11 4b 00 00 1e 06 - 68 4b 81 20 01 40 80 c1      | @ K    hK   @  |

0020: 20 01 0a 1e 00 15 a2 a4 - c2 34 02 91 6e b2 50 18      |         4  n P |

0030: 10 00 44 dc 00 00 50 4f - 52 54 20 31 32 39 2c 33      |  D   PORT 129,3|

0040: 32 2c 31 2c 36 34 2c 31 - 30 2c 33 31 0d 0a            |2,1,64,10,31  |

Packet 21 - shirley to cs - REPEAT, use port 10*256+31=2591 when you send data

Receive time:  100.585   packet length: 78     received length: 78

Ethernet:   (00000f007ed9 -> Prote e07055)  type: IP(0x0800)

Internet:     129.32.1.64 -> 128.193.32.1    hl: 5   ver: 4   tos: 0

len: 64   id: 0x114c fragoff: 0   flags: 00 ttl: 30  prot: TCP(6)

xsum: 0x4a68

TCP:            2590 -> ftp(21)        seq: a2a4c234  ack: 02916eb2

win: 4096   hl: 5   xsum: 0xdc44  urg: 0      flags: <ACK><PUSH>

data (24/24): PORT 129,32,1,64,10,31

0000: 00 00 93 e0 70 55 00 00 - 0f 00 7e d9 08 00 45 00      |    pU    ~   E |

0010: 00 40 11 4c 00 00 1e 06 - 68 4a 81 20 01 40 80 c1      | @ L    hJ   @  |

0020: 20 01 0a 1e 00 15 a2 a4 - c2 34 02 91 6e b2 50 18      |         4  n P |

0030: 10 00 44 dc 00 00 50 4f - 52 54 20 31 32 39 2c 33      |  D   PORT 129,3|

0040: 32 2c 31 2c 36 34 2c 31 - 30 2c 33 31 0d 0a            |2,1,64,10,31  |

Packet 22 - cs to shirley - "200 PORT command okay."

Receive time:  100.958   packet length: 78     received length: 78

Ethernet:   (Prote e07055 -> 00000f007ed9)  type: IP(0x0800)

Internet:    128.193.32.1 -> 129.32.1.64     hl: 5   ver: 4   tos: 0

len: 64   id: 0x8b95 fragoff: 0   flags: 00 ttl: 13  prot: TCP(6)

xsum: 0x00ff

TCP:         ftp(21) -> 2590           seq: 02916eb2  ack: a2a4c24c

win: 4096   hl: 5   xsum: 0x9719  urg: 0      flags: <ACK><PUSH>

data (24/24): 200 PORT command okay.

0000: 00 00 0f 00 7e d9 00 00 - 93 e0 70 55 08 00 45 00      |    ~     pU  E |

0010: 00 40 8b 95 00 00 0d 06 - ff 00 80 c1 20 01 81 20      | @              |

0020: 01 40 00 15 0a 1e 02 91 - 6e b2 a2 a4 c2 4c 50 18      | @      n    LP |

0030: 10 00 19 97 00 00 32 30 - 30 20 50 4f 52 54 20 63      |      200 PORT c|

0040: 6f 6d 6d 61 6e 64 20 6f - 6b 61 79 2e 0d 0a            |ommand okay.  |

Packet 23 - shirley to cs - "LIST"

Receive time:  100.960   packet length: 60     received length: 60

Ethernet:   (00000f007ed9 -> Prote e07055)  type: IP(0x0800)

Internet:     129.32.1.64 -> 128.193.32.1    hl: 5   ver: 4   tos: 0

len: 46   id: 0x114d fragoff: 0   flags: 00 ttl: 30  prot: TCP(6)

xsum: 0x5b68

TCP:            2590 -> ftp(21)        seq: a2a4c24c  ack: 02916eca

win: 4096   hl: 5   xsum: 0x7cef  urg: 0      flags: <ACK><PUSH>

data (6/6): LIST

0000: 00 00 93 e0 70 55 00 00 - 0f 00 7e d9 08 00 45 00      |    pU    ~   E |

0010: 00 2e 11 4d 00 00 1e 06 - 68 5b 81 20 01 40 80 c1      | . M    h[   @  |

0020: 20 01 0a 1e 00 15 a2 a4 - c2 4c 02 91 6e ca 50 18      |         L  n P |

0030: 10 00 ef 7c 00 00 4c 49 - 53 54 0d 0a                  |   |  LIST  |

Packet 24 - cs to shirley - "150 Opening data connection to your port 2591"

Receive time:  101.348   packet length: 128    received length: 128

Ethernet:   (Prote e07055 -> 00000f007ed9)  type: IP(0x0800)

Internet:    128.193.32.1 -> 129.32.1.64     hl: 5   ver: 4   tos: 0

len: 114  id: 0x8b9d fragoff: 0   flags: 00 ttl: 13  prot: TCP(6)

xsum: 0xc6fe

TCP:         ftp(21) -> 2590           seq: 02916eca  ack: a2a4c252

win: 4096   hl: 5   xsum: 0x3921  urg: 0      flags: <ACK><PUSH>

data (60/74): 150 Opening data connection for /bin/ls -l (129.32.1.64,2591

0000: 00 00 0f 00 7e d9 00 00 - 93 e0 70 55 08 00 45 00      |    ~     pU  E |

0010: 00 72 8b 9d 00 00 0d 06 - fe c6 80 c1 20 01 81 20      | r              |

0020: 01 40 00 15 0a 1e 02 91 - 6e ca a2 a4 c2 52 50 18      | @      n    RP |

0030: 10 00 21 39 00 00 31 35 - 30 20 4f 70 65 6e 69 6e      |  !9  150 Openin|

0040: 67 20 64 61 74 61 20 63 - 6f 6e 6e 65 63 74 69 6f      |g data connectio|

0050: 6e 20 66 6f 72 20 2f 62 - 69 6e 2f 6c 73 20 2d 6c      |n for /bin/ls -l|

0060: 20 28 31 32 39 2e 33 32 - 2e 31 2e 36 34 2c 32 35      | (129.32.1.64,25|

0070: 39 31 29 20 28 30 20 62 - 79 74 65 73 29 2e 0d 0a      |91) (0 bytes).  |

Packets 25, 26, and 28 represent the three-way handshake to open a second

conversation (the data channel) to transfer the requested data. This is

a separate conversation with its own sequence and acknowledgement numbers.

Packet 25 - cs to shirley (on data channel) - My seq numbers begin at 02bc6600

Receive time:  101.354   packet length: 60     received length: 60

Ethernet:   (Prote e07055 -> 00000f007ed9)  type: IP(0x0800)

Internet:    128.193.32.1 -> 129.32.1.64     hl: 5   ver: 4   tos: 0

len: 40   id: 0x8b9e fragoff: 0   flags: 00 ttl: 13  prot: TCP(6)

xsum: 0x0fff

TCP:    ftp-data(20) -> 2591           seq: 02bc6600  ack:   ----

win: 4096   hl: 5   xsum: 0xd109  urg: 0      flags: <SYN>

0000: 00 00 0f 00 7e d9 00 00 - 93 e0 70 55 08 00 45 00      |    ~     pU  E |

0010: 00 28 8b 9e 00 00 0d 06 - ff 0f 80 c1 20 01 81 20      | (              |

0020: 01 40 00 14 0a 1f 02 bc - 66 00 00 00 00 00 50 02      | @      f     P |

0030: 10 00 09 d1 00 00 40 60 - 52 54 20 31                  |      @`RT 1|

Packet 26 - shirley to cs on data channel - OK, and mine begin at a2cbd201

Receive time:  101.355   packet length: 60     received length: 60

Ethernet:   (00000f007ed9 -> Prote e07055)  type: IP(0x0800)

Internet:     129.32.1.64 -> 128.193.32.1    hl: 5   ver: 4   tos: 0

len: 40   id: 0x114e fragoff: 0   flags: 00 ttl: 30  prot: TCP(6)

xsum: 0x6068

TCP:            2591 -> ftp-data(20)   seq: a2cbd201  ack: 02bc6601

win: 4096   hl: 5   xsum: 0xf294  urg: 0      flags: <ACK><SYN>

0000: 00 00 93 e0 70 55 00 00 - 0f 00 7e d9 08 00 45 00      |    pU    ~   E |

0010: 00 28 11 4e 00 00 1e 06 - 68 60 81 20 01 40 80 c1      | ( N    h`   @  |

0020: 20 01 0a 1f 00 14 a2 cb - d2 01 02 bc 66 01 50 12      |            f P |

0030: 10 00 94 f2 00 00 40 60 - 52 54 20 31                  |      @`RT 1|

Packet 27 - shirley to cs - OK (NOTE - this is the ack of packet 24!)

Receive time:  101.525   packet length: 60     received length: 60

Ethernet:   (00000f007ed9 -> Prote e07055)  type: IP(0x0800)

Internet:     129.32.1.64 -> 128.193.32.1    hl: 5   ver: 4   tos: 0

len: 40   id: 0x114f fragoff: 0   flags: 00 ttl: 30  prot: TCP(6)

xsum: 0x5f68

TCP:            2590 -> ftp(21)        seq: a2a4c252  ack: 02916f14

win: 4096   hl: 5   xsum: 0xe29b  urg: 0      flags: <ACK>

0000: 00 00 93 e0 70 55 00 00 - 0f 00 7e d9 08 00 45 00      |    pU    ~   E |

0010: 00 28 11 4f 00 00 1e 06 - 68 5f 81 20 01 40 80 c1      | ( O    h_   @  |

0020: 20 01 0a 1e 00 15 a2 a4 - c2 52 02 91 6f 14 50 10      |         R  o P |

0030: 10 00 9b e2 00 00 40 60 - 52 54 20 31                  |      @`RT 1|

Packet 28 - cs to shirley on data channel- ok

Receive time:  101.725   packet length: 60     received length: 60

Ethernet:   (Prote e07055 -> 00000f007ed9)  type: IP(0x0800)

Internet:    128.193.32.1 -> 129.32.1.64     hl: 5   ver: 4   tos: 0

len: 40   id: 0x8ba4 fragoff: 0   flags: 00 ttl: 13  prot: TCP(6)

xsum: 0x09ff

TCP:    ftp-data(20) -> 2591           seq: 02bc6601  ack: a2cbd202

win: 4096   hl: 5   xsum: 0xf394  urg: 0      flags: <ACK>

0000: 00 00 0f 00 7e d9 00 00 - 93 e0 70 55 08 00 45 00      |    ~     pU  E |

0010: 00 28 8b a4 00 00 0d 06 - ff 09 80 c1 20 01 81 20      | (              |

0020: 01 40 00 14 0a 1f 02 bc - 66 01 a2 cb d2 02 50 10      | @      f     P |

0030: 10 00 94 f3 00 00 40 60 - 0a 12 40 60                  |      @`  @`|

Packet 29 - cs to shirley on data channel - TCP goodby (1/3 of 3-way handshake)

Receive time:  101.796   packet length: 60     received length: 60

Ethernet:   (Prote e07055 -> 00000f007ed9)  type: IP(0x0800)

Internet:    128.193.32.1 -> 129.32.1.64     hl: 5   ver: 4   tos: 0

len: 40   id: 0x8ba6 fragoff: 0   flags: 00 ttl: 13  prot: TCP(6)

xsum: 0x07ff

TCP:    ftp-data(20) -> 2591           seq: 02bc66bb  ack: a2cbd202

win: 4096   hl: 5   xsum: 0x3894  urg: 0      flags: <ACK><FIN>

0000: 00 00 0f 00 7e d9 00 00 - 93 e0 70 55 08 00 45 00      |    ~     pU  E |

0010: 00 28 8b a6 00 00 0d 06 - ff 07 80 c1 20 01 81 20      | (              |

0020: 01 40 00 14 0a 1f 02 bc - 66 bb a2 cb d2 02 50 11      | @      f     P |

0030: 10 00 94 38 00 00 40 60 - 52 54 20 31                  |   8  @`RT 1|

Packets 29 and 30 have been delivered out of order (check the sequence number)!

Packet 30 - cs to shirley on data channel - (the directory information)

Receive time:  101.836   packet length: 240    received length: 240

Ethernet:   (Prote e07055 -> 00000f007ed9)  type: IP(0x0800)

Internet:    128.193.32.1 -> 129.32.1.64     hl: 5   ver: 4   tos: 0

len: 226  id: 0x8ba5 fragoff: 0   flags: 00 ttl: 13  prot: TCP(6)

xsum: 0x4efe

TCP:    ftp-data(20) -> 2591           seq: 02bc6601  ack: a2cbd202

win: 4096   hl: 5   xsum: 0xe4fc  urg: 0      flags: <ACK><PUSH>

data (60/186): total 6

dr-xr-xr-x   2 root     root        1024 Oct 15  19

0000: 00 00 0f 00 7e d9 00 00 - 93 e0 70 55 08 00 45 00      |    ~     pU  E |

0010: 00 e2 8b a5 00 00 0d 06 - fe 4e 80 c1 20 01 81 20      |         N      |

0020: 01 40 00 14 0a 1f 02 bc - 66 01 a2 cb d2 02 50 18      | @      f     P |

0030: 10 00 fc e4 00 00 74 6f - 74 61 6c 20 36 0d 0a 64      |      total 6  d|

0040: 72 2d 78 72 2d 78 72 2d - 78 20 20 20 32 20 72 6f      |r-xr-xr-x   2 ro|

0050: 6f 74 20 20 20 20 20 72 - 6f 6f 74 20 20 20 20 20      |ot     root     |

0060: 20 20 20 31 30 32 34 20 - 4f 63 74 20 31 35 20 20      |   1024 Oct 15  |

0070: 31 39 39 30 20 62 69 6e - 0d 0a 64 72 2d 78 72 2d      |1990 bin  dr-xr-|

0080: 78 72 2d 78 20 20 20 32 - 20 72 6f 6f 74 20 20 20      |xr-x   2 root   |

0090: 20 20 72 6f 6f 74 20 20 - 20 20 20 20 20 20 31 30      |  root        10|

00a0: 32 34 20 4a 75 6e 20 31 - 33 20 31 36 3a 32 32 20      |24 Jun 13 16:22 |

00b0: 65 74 63 0d 0a 64 72 77 - 78 72 2d 78 72 2d 78 20      |etc  drwxr-xr-x |

00c0: 20 32 34 20 72 6f 6f 74 - 20 20 20 20 20 73 79 73      | 24 root     sys|

00d0: 20 20 20 20 20 20 20 20 - 20 31 30 32 34 20 53 65      |         1024 Se|

00e0: 70 20 31 30 20 31 39 3a - 35 35 20 70 75 62 0d 0a      |p 10 19:55 pub  |

Packet 31 - shirley to cs on data channel - ok

Receive time:  101.837   packet length: 60     received length: 60

Ethernet:   (00000f007ed9 -> Prote e07055)  type: IP(0x0800)

Internet:     129.32.1.64 -> 128.193.32.1    hl: 5   ver: 4   tos: 0

len: 40   id: 0x1150 fragoff: 0   flags: 00 ttl: 30  prot: TCP(6)

xsum: 0x5e68

TCP:            2591 -> ftp-data(20)   seq: a2cbd202  ack: 02bc66bc

win: 3910   hl: 5   xsum: 0xf294  urg: 0      flags: <ACK>

0000: 00 00 93 e0 70 55 00 00 - 0f 00 7e d9 08 00 45 00      |    pU    ~   E |

0010: 00 28 11 50 00 00 1e 06 - 68 5e 81 20 01 40 80 c1      | ( P    h^   @  |

0020: 20 01 0a 1f 00 14 a2 cb - d2 02 02 bc 66 bc 50 10      |            f P |

0030: 0f 46 94 f2 00 00 02 63 - 73 04 6f 72                  | F     cs or|

Packet 32 - shirley to cs on data channel - TCP goodby (2/3 of 3-way handshake)

Receive time:  101.839   packet length: 60     received length: 60

Ethernet:   (00000f007ed9 -> Prote e07055)  type: IP(0x0800)

Internet:     129.32.1.64 -> 128.193.32.1    hl: 5   ver: 4   tos: 0

len: 40   id: 0x1151 fragoff: 0   flags: 00 ttl: 30  prot: TCP(6)

xsum: 0x5d68

TCP:            2591 -> ftp-data(20)   seq: a2cbd202  ack: 02bc66bc

win: 4096   hl: 5   xsum: 0x3794  urg: 0      flags: <ACK><FIN>

0000: 00 00 93 e0 70 55 00 00 - 0f 00 7e d9 08 00 45 00      |    pU    ~   E |

0010: 00 28 11 51 00 00 1e 06 - 68 5d 81 20 01 40 80 c1      | ( Q    h]   @  |

0020: 20 01 0a 1f 00 14 a2 cb - d2 02 02 bc 66 bc 50 11      |            f P |

0030: 10 00 94 37 00 00 74 6f - 74 61 6c 20                  |   7  total |

Packet 33 - cs to shirley - "226 Transfer complete."

Receive time:  101.847   packet length: 78     received length: 78

Ethernet:   (Prote e07055 -> 00000f007ed9)  type: IP(0x0800)

Internet:    128.193.32.1 -> 129.32.1.64     hl: 5   ver: 4   tos: 0

len: 64   id: 0x8ba7 fragoff: 0   flags: 00 ttl: 13  prot: TCP(6)

xsum: 0xeefe

TCP:         ftp(21) -> 2590           seq: 02916f14  ack: a2a4c252

win: 4096   hl: 5   xsum: 0xcccc  urg: 0      flags: <ACK><PUSH>

data (24/24): 226 Transfer complete.

0000: 00 00 0f 00 7e d9 00 00 - 93 e0 70 55 08 00 45 00      |    ~     pU  E |

0010: 00 40 8b a7 00 00 0d 06 - fe ee 80 c1 20 01 81 20      | @              |

0020: 01 40 00 15 0a 1e 02 91 - 6f 14 a2 a4 c2 52 50 18      | @      o    RP |

0030: 10 00 cc cc 00 00 32 32 - 36 20 54 72 61 6e 73 66      |      226 Transf|

0040: 65 72 20 63 6f 6d 70 6c - 65 74 65 2e 0d 0a            |er complete.  |

Packet 34 - shirley to cs - ok

Receive time:  101.901   packet length: 60     received length: 60

Ethernet:   (00000f007ed9 -> Prote e07055)  type: IP(0x0800)

Internet:     129.32.1.64 -> 128.193.32.1    hl: 5   ver: 4   tos: 0

len: 40   id: 0x1152 fragoff: 0   flags: 00 ttl: 30  prot: TCP(6)

xsum: 0x5c68

TCP:            2590 -> ftp(21)        seq: a2a4c252  ack: 02916f2c

win: 4096   hl: 5   xsum: 0xca9b  urg: 0      flags: <ACK>

0000: 00 00 93 e0 70 55 00 00 - 0f 00 7e d9 08 00 45 00      |    pU    ~   E |

0010: 00 28 11 52 00 00 1e 06 - 68 5c 81 20 01 40 80 c1      | ( R    h\   @  |

0020: 20 01 0a 1e 00 15 a2 a4 - c2 52 02 91 6f 2c 50 10      |         R  o,P |

0030: 10 00 9b ca 00 00 32 32 - 36 20 54 72                  |      226 Tr|

Packet 35 - cs to shirley on data channel - TCP goodby (3/3 of 3-way handshake)

Receive time:  102.173   packet length: 60     received length: 60

Ethernet:   (Prote e07055 -> 00000f007ed9)  type: IP(0x0800)

Internet:    128.193.32.1 -> 129.32.1.64     hl: 5   ver: 4   tos: 0

len: 40   id: 0x8baa fragoff: 0   flags: 00 ttl: 13  prot: TCP(6)

xsum: 0x03ff

TCP:    ftp-data(20) -> 2591           seq: 02bc66bc  ack: a2cbd203

win: 4096   hl: 5   xsum: 0x3794  urg: 0      flags: <ACK>

0000: 00 00 0f 00 7e d9 00 00 - 93 e0 70 55 08 00 45 00      |    ~     pU  E |

0010: 00 28 8b aa 00 00 0d 06 - ff 03 80 c1 20 01 81 20      | (              |

0020: 01 40 00 14 0a 1f 02 bc - 66 bc a2 cb d2 03 50 10      | @      f     P |

0030: 10 00 94 37 00 00 40 60 - 0a 12 40 60                  |   7  @`  @`|

Packet 36 - shirley to cs - "QUIT" (Must close FTP conversation first)

Receive time:  104.091   packet length: 60     received length: 60

Ethernet:   (00000f007ed9 -> Prote e07055)  type: IP(0x0800)

Internet:     129.32.1.64 -> 128.193.32.1    hl: 5   ver: 4   tos: 0

len: 46   id: 0x1154 fragoff: 0   flags: 00 ttl: 30  prot: TCP(6)

xsum: 0x5468

TCP:            2590 -> ftp(21)        seq: a2a4c252  ack: 02916f2c

win: 4096   hl: 5   xsum: 0x08f4  urg: 0      flags: <ACK><PUSH>

data (6/6): QUIT

0000: 00 00 93 e0 70 55 00 00 - 0f 00 7e d9 08 00 45 00      |    pU    ~   E |

0010: 00 2e 11 54 00 00 1e 06 - 68 54 81 20 01 40 80 c1      | . T    hT   @  |

0020: 20 01 0a 1e 00 15 a2 a4 - c2 52 02 91 6f 2c 50 18      |         R  o,P |

0030: 10 00 f4 08 00 00 51 55 - 49 54 0d 0a                  |      QUIT  |

Packet 37 - cs to shirley - "221 Goodbye."

Receive time:  104.316   packet length: 68     received length: 68

Ethernet:   (Prote e07055 -> 00000f007ed9)  type: IP(0x0800)

Internet:    128.193.32.1 -> 129.32.1.64     hl: 5   ver: 4   tos: 0

len: 54   id: 0x8bc9 fragoff: 0   flags: 00 ttl: 13  prot: TCP(6)

xsum: 0xd6fe

TCP:         ftp(21) -> 2590           seq: 02916f2c  ack: a2a4c258

win: 4096   hl: 5   xsum: 0xd6ac  urg: 0      flags: <ACK><PUSH>

data (14/14): 221 Goodbye.

0000: 00 00 0f 00 7e d9 00 00 - 93 e0 70 55 08 00 45 00      |    ~     pU  E |

0010: 00 36 8b c9 00 00 0d 06 - fe d6 80 c1 20 01 81 20      | 6              |0020: 01

40 00 15 0a 1e 02 91 - 6f 2c a2 a4 c2 58 50 18      | @      o,   XP |

0030: 10 00 ac d6 00 00 32 32 - 31 20 47 6f 6f 64 62 79      |      221 Goodby|

0040: 65 2e 0d 0a                                            |e.  |

Packet 38 - cs to shirley - TCP goodbye (1/3 of 3-way handshake)

Receive time:  104.317   packet length: 60     received length: 60

Ethernet:   (Prote e07055 -> 00000f007ed9)  type: IP(0x0800)

Internet:    128.193.32.1 -> 129.32.1.64     hl: 5   ver: 4   tos: 0

len: 40   id: 0x8bca fragoff: 0   flags: 00 ttl: 13  prot: TCP(6)

xsum: 0xe3fe

TCP:         ftp(21) -> 2590           seq: 02916f3a  ack: a2a4c258

win: 4096   hl: 5   xsum: 0xb59b  urg: 0      flags: <ACK><FIN>

0000: 00 00 0f 00 7e d9 00 00 - 93 e0 70 55 08 00 45 00      |    ~     pU  E |

0010: 00 28 8b ca 00 00 0d 06 - fe e3 80 c1 20 01 81 20      | (              |

0020: 01 40 00 15 0a 1e 02 91 - 6f 3a a2 a4 c2 58 50 11      | @      o:   XP |

0030: 10 00 9b b5 00 00 40 60 - 0a 12 40 60                  |      @`  @`|

Packet 39 - shirley to cs - ok

Receive time:  104.318   packet length: 60     received length: 60

Ethernet:   (00000f007ed9 -> Prote e07055)  type: IP(0x0800)

Internet:     129.32.1.64 -> 128.193.32.1    hl: 5   ver: 4   tos: 0

len: 40   id: 0x1155 fragoff: 0   flags: 00 ttl: 30  prot: TCP(6)

xsum: 0x5968

TCP:            2590 -> ftp(21)        seq: a2a4c258  ack: 02916f3b

win: 4096   hl: 5   xsum: 0xb59b  urg: 0      flags: <ACK>

0000: 00 00 93 e0 70 55 00 00 - 0f 00 7e d9 08 00 45 00      |    pU    ~   E |

0010: 00 28 11 55 00 00 1e 06 - 68 59 81 20 01 40 80 c1      | ( U    hY   @  |

0020: 20 01 0a 1e 00 15 a2 a4 - c2 58 02 91 6f 3b 50 10      |         X  o;P |

0030: 10 00 9b b5 00 00 40 60 - 0a 12 40 60                  |      @`  @`|

Packet 40 - shirley to cs - TCP goodbye (2/3 of 3-way handshake)

Receive time:  104.376   packet length: 60     received length: 60

Ethernet:   (00000f007ed9 -> Prote e07055)  type: IP(0x0800)

Internet:     129.32.1.64 -> 128.193.32.1    hl: 5   ver: 4   tos: 0

len: 40   id: 0x1156 fragoff: 0   flags: 00 ttl: 30  prot: TCP(6)

xsum: 0x5868

TCP:            2590 -> ftp(21)        seq: a2a4c258  ack: 02916f3b

win: 4096   hl: 5   xsum: 0xb49b  urg: 0      flags: <ACK><FIN>

0000: 00 00 93 e0 70 55 00 00 - 0f 00 7e d9 08 00 45 00      |    pU    ~   E |

0010: 00 28 11 56 00 00 1e 06 - 68 58 81 20 01 40 80 c1      | ( V    hX   @  |

0020: 20 01 0a 1e 00 15 a2 a4 - c2 58 02 91 6f 3b 50 11      |         X  o;P |

0030: 10 00 9b b4 00 00 40 60 - 0a 12 40 60                  |      @`  @`|

Packet 41 - shirley to cs - DUPLICATE TCP goodbye (2/3 of 3-way handshake)

Receive time:  105.598   packet length: 60     received length: 60

Ethernet:   (00000f007ed9 -> Prote e07055)  type: IP(0x0800)

Internet:     129.32.1.64 -> 128.193.32.1    hl: 5   ver: 4   tos: 0

len: 40   id: 0x1157 fragoff: 0   flags: 00 ttl: 30  prot: TCP(6)

xsum: 0x5768

TCP:            2590 -> ftp(21)        seq: a2a4c258  ack: 02916f3b

win: 4096   hl: 5   xsum: 0xb49b  urg: 0      flags: <ACK><FIN>

0000: 00 00 93 e0 70 55 00 00 - 0f 00 7e d9 08 00 45 00      |    pU    ~   E |

0010: 00 28 11 57 00 00 1e 06 - 68 57 81 20 01 40 80 c1      | ( W    hW   @  |

0020: 20 01 0a 1e 00 15 a2 a4 - c2 58 02 91 6f 3b 50 11      |         X  o;P |

0030: 10 00 9b b4 00 00 70 69 - 63 61 73 73                  |      picass|

Packet 42 - cs to shirley - TCP goodbye (3/3 of 3-way handshake)

Receive time:  105.849   packet length: 60     received length: 60

Ethernet:   (Prote e07055 -> 00000f007ed9)  type: IP(0x0800)

Internet:    128.193.32.1 -> 129.32.1.64     hl: 5   ver: 4   tos: 0

len: 40   id: 0x8bee fragoff: 0   flags: 00 ttl: 13  prot: TCP(6)

xsum: 0xbffe

TCP:         ftp(21) -> 2590           seq: 02916f3b  ack: a2a4c259

win: 4096   hl: 5   xsum: 0xb49b  urg: 0      flags: <ACK>

0000: 00 00 0f 00 7e d9 00 00 - 93 e0 70 55 08 00 45 00      |    ~     pU  E |

0010: 00 28 8b ee 00 00 0d 06 - fe bf 80 c1 20 01 81 20      | (              |

0020: 01 40 00 15 0a 1e 02 91 - 6f 3b a2 a4 c2 59 50 10      | @      o;   YP |

0030: 10 00 9b b4 00 00 40 60 - 02 18 00 00                  |      @`    |